NHAI 6th Innovation Hackathon ReflectAI AI-Powered Retroreflectivity Intelligence Platform Concept Note & Architecture Document

| Hackathon | NHAI 6th Innovation Hackathon |
| --- | --- |
| Problem Domain | Retroreflectivity Measurement — Signs, Pavement Markings & Road Studs |
| Approach | Option iii — Vehicle-Mounted Sensors + AI/ML/CV Fusion |
| Submission Type | Concept Note + Architecture Document |
| Submission Date | 23 April 2026 |

| 01 | Executive Summary |
| --- | --- |

India's National Highways span over 1,45,000 km, hosting millions of signs, pavement markings, delineators, and road studs. Ensuring that all retroreflective elements meet IRC 67/IRC 35 standards is a non-negotiable safety requirement — yet the current method of manual, handheld retroreflectometer measurement is dangerously slow, expensive, and exposes inspection crews to live high-speed traffic.

We propose ReflectAI — a hybrid, AI-powered retroreflectivity intelligence platform that combines vehicle-mounted multi-spectral sensors with deep learning to deliver continuous, real-time retroreflectivity assessment across the entire NHAI road network, at highway speed, without disrupting traffic.

Core Value Proposition ReflectAI reduces inspection time by ~95%, eliminates manual roadside crew exposure, delivers sub-metre GPS-tagged reflectivity heatmaps, and predicts degradation 6–18 months in advance — enabling proactive, budgeted maintenance.

### Why Option iii is the Winning Choice

NHAI provides three options. Option i (vehicle/drone hardware only) achieves speed but not intelligence. Option ii (AI/ML only) achieves intelligence but not real-time physical accuracy. Option iii — the combination — is the only approach that achieves both simultaneously, and is the only approach that can produce the legally defensible quantitative mcd/m²/lux readings that IRC standards demand. It is also where the global research frontier currently sits (MIT, TxDOT, FHWA 2024–2026).

| 02 | Problem Deep-Dive & Gap Analysis |
| --- | --- |

## 2.1  What is Retroreflectivity?

Retroreflectivity (measured in mcd/m²/lux, or RL) describes a surface's ability to return light directly back to its source. Road signs and pavement markings use micro-prism or glass-bead sheeting to return headlight beams to the driver's eyes, creating the 'glow' effect that guides night driving. As this material ages, RL drops below safe thresholds.

## 2.2  Current Measurement Approach & Its Failures

| Dimension | Handheld Device (Current) | ReflectAI (Proposed) |
| --- | --- | --- |
| Speed | ~5–8 readings/hour per crew | 10–15 km/min at highway speed |
| Coverage | Spot/sampling only | 100% continuous coverage |
| Safety Risk | Crew exposed to live traffic | Zero crew road exposure |
| Night Operation | Required; dangerous | Full day + night capability |
| Asset Types | One type per session | Signs, markings & studs simultaneously |
| Weather Dependency | Affected by wet/fog | Multi-condition adaptive model |
| Data Output | Paper/manual log | GPS-tagged GIS heatmap + dashboard |
| Predictive Maintenance | None | 6–18 month degradation forecast |
| Cost per km | High (crew + time) | ~70–80% reduction estimated |

## 2.3  Scope of the Problem on NHAI Network

- 8-lane expressways, 6-lane, 4-lane, and 2-lane highways across 1,45,000+ km

- Assets include: gantry signs (overhead), shoulder-mounted signs, centre-line markings, edge markings, road studs (RPMs), delineators

- Signs and markings must meet IRC 67 (signs) and IRC 35 (road markings) standards

- Measurement required at installation and periodically during O&M lifecycle

- Deterioration caused by: exhaust fumes, tyre friction, UV exposure, fog, dirt, monsoon washing

| 03 | Proposed Solution — ReflectAI |
| --- | --- |

## 3.1  Solution Philosophy

ReflectAI is built on three principles: (1) Measure everything, not just samples; (2) Measure at the speed of traffic, not the speed of walking; and (3) Turn measurements into decisions, not just data.

The platform fuses two complementary sensing modalities — active photometric measurement (near-infrared LED illumination + calibrated camera) and passive computer vision (neural network analysis of day/night imagery) — to produce absolute RL values that can be cross-validated, GPS-mapped, and trend-analyzed.

## 3.2  Three-Mode Operation

| Mode | Configuration | Best For |
| --- | --- | --- |
| A — Survey Vehicle | Survey van @ 60–80 km/h with full sensor rig. Covers 500+ km/day. | Expressways, 6-lane, 4-lane highways — primary O&M audit |
| B — Drone (UAV) | Fixed-wing or quad drone with NIR-illumination payload. Covers gantry signs, remote stretches, and post-disaster roads. | Overhead gantry signs, inaccessible stretches, post-event rapid assessment |
| C — Edge Camera | Fixed roadside AI camera with calibrated light source for continuous spot monitoring. | High-risk zones (hill roads, fog zones, accident blackspots) — continuous monitoring |

| 04 | System Architecture |
| --- | --- |

## 4.1  Architecture Overview — Five Layers

ReflectAI is organized as five vertical layers, each independently deployable and upgradable.

LAYER 1 — DATA ACQUISITION (Hardware Rig)

Vehicle Sensor Stack (roof-mounted, weather-sealed enclosure): - Stereo HDR Camera Array (2× cameras, 4K, 120fps, wide dynamic range) — captures luminance from both pavement and sign surfaces simultaneously - Calibrated NIR-LED Illumination Bar — forward-facing, fixed geometry, simulates standardized observation angle per EN 1436 / IRC 67 for pavement; adjustable angle bracket for signs - High-Precision GPS/IMU Unit (RTK-GPS, ±2 cm accuracy) — tags every frame with sub-metre coordinates, speed, heading, and road gradient - Ambient Light Sensor + Weather Station Module — real-time Lux, temperature, humidity, rain, fog detection for condition-aware normalization - Optional: Solid-State LiDAR (16-channel) — provides point cloud intensity for direct RL correlation on pavement markings (proven to R² = 0.97 in literature) Drone Payload: Lightweight variant of above — NIR LED ring + single global shutter camera + GPS/IMU. Used for gantry signs and inaccessible locations.

LAYER 2 — EDGE COMPUTING (On-vehicle Real-Time Processing)

NVIDIA Jetson Orin NX (or equivalent edge GPU) running: - Asset Detection Module — YOLOv9 / RT-DETR model fine-tuned on Indian road sign shapes (triangle, circle, rectangle, gantry) and pavement marking types (solid white/yellow, dashed, edge line, zebra, word markings) - RL Estimation Module — CNN regression head that maps pixel luminance + geometry + illumination metadata → absolute RL value in mcd/m²/lux. Calibrated against ground truth handheld measurements - Condition Normalizer — corrects raw readings for ambient light, wet/dry surface, fog density, and vehicle speed using trained regression models - Asset Classifier — identifies asset type, color, material class, and maps it to applicable IRC threshold (e.g., white centre line ≥ 100 mcd/m²/lux, Engineer Grade sign ≥ 70) - Local SSD buffer — stores raw frames + processed results for 5 days of survey before cloud sync. Operates fully offline in tunnel/no-signal zones.

LAYER 3 — AI/ML INTELLIGENCE ENGINE (Cloud / On-premise)

- Degradation Prediction Model — Random Forest / Gradient Boosting ensemble trained on historical RL measurements, material type, age, traffic volume, weather, and material class. Predicts RL at T+6, T+12, T+18 months. (Literature benchmark: R² = 0.97 with random forest on similar data) - Anomaly Detection — Unsupervised model that flags statistically unusual RL drops (sudden vandalism, accident damage, material failure) vs. expected gradual wear - Maintenance Priority Scorer — Multi-factor model combining current RL, predicted RL trajectory, traffic ADT, accident history, and asset replacement cost to produce a ranked maintenance queue - Calibration Engine — Continuously improves RL estimation accuracy by ingesting new ground-truth handheld measurements and retraining edge models via OTA update - Multi-Condition Model Variants — separate model weights for Dry Day, Dry Night, Wet Day, Wet Night, Fog — auto-selected based on real-time weather sensor input

LAYER 4 — DATA PLATFORM (Storage, Integration, APIs)

- Time-Series Asset Database — every asset (sign / marking segment / stud) has a time-series RL history, GPS coordinates, photos, classification labels, and IRC compliance status - GIS Layer — integrates with NHAI's existing GIS infrastructure. Exports RL heatmaps as standard GeoJSON / Shapefile layers compatible with QGIS and ArcGIS - REST API — allows integration with NHAI's ATMS, O&M contractor portals, and third-party maintenance management systems - Audit-Ready Export — auto-generates IRC-compliant inspection reports with GPS coordinates, timestamps, measurement values, and pass/fail status for each asset

LAYER 5 — COMMAND DASHBOARD (Web + Mobile)

- Interactive Map Dashboard — RL heatmap overlaid on highway map. Color-coded by IRC compliance (Green: OK, Amber: Caution, Red: Below threshold). Drill-down to individual assets with photo, history, and predicted decline curve - Maintenance Work Queue — auto-prioritized list of assets needing attention. Assignable to O&M contractors with deadline, GPS location, and specification sheet - Survey Mission Planner — schedule vehicle and drone survey routes with estimated coverage, time, and cost - Mobile Field App — O&M field crew can view assigned work orders, capture post-remediation photos, and log verification measurements from handheld devices

| 05 | AI/ML & Computer Vision Pipeline |
| --- | --- |

## 5.1  Data Ingestion & Pre-Processing

- Raw video frames captured at 120fps, synchronized with GPS/IMU timestamps (±1ms)

- Frame selection: intelligent keyframe extraction at every 0.5m distance traveled (not time-based, avoiding blur at speed variations)

- Geometric distortion correction via camera calibration matrix (Zhang method)

- Illumination normalization using ambient light sensor reading and known NIR LED output curve

## 5.2  Asset Detection — Object Detection Model

- Base Model: YOLOv9 / RT-DETR fine-tuned on custom Indian highway dataset

- Training data: 50,000+ annotated frames from Indian highways covering sign types per IRC 67, all pavement marking classes per IRC 35, road studs and delineators

- Multi-class output: bounding box + class label + confidence score + distance estimate

- Sign detection operates on 'looking forward' camera; marking detection on downward-facing camera — parallel inference pipelines

## 5.3  Retroreflectivity Estimation — Core Innovation

This is the heart of ReflectAI's differentiation. We use a dual-path estimation approach:

| Path A: Physical Photometric Model | Path B: Deep Learning Regression |
| --- | --- |
| Computes RL from first principles: - Measured pixel luminance (L, cd/m²) from calibrated camera - Known illuminance (E, lux) from LED array at measured distance - Geometry correction: observation angle, entrance angle, vehicle speed correction RL = L / E × 1000 (mcd/m²/lux) | CNN regression network that learns: - Pixel intensity distribution within detected bounding box - Surface texture features (glassbeads vs. prismatic) - Color channel ratios and HDR response curves Outputs RL estimate → cross-validated with Path A. Fusion via weighted ensemble. |

## 5.4  Condition Adaptation — All-Weather Operation

- Wet surface: separate model weights for wet road (measured via rain sensor + surface brightness ratio); RL typically drops 30–50% on wet pavement markings — model accounts for this

- Night/day: active NIR illumination ensures consistent geometry regardless of ambient light; model switches to 'night' weights below 50 lux ambient

- Fog: fog density estimated from NIR camera scatter coefficient; applies atmospheric transmittance correction to measured luminance

- Streetlit vs. unlit: ambient light sensor continuously adjusts the illuminance denominator in the photometric model

## 5.5  Predictive Degradation Model

- Inputs: initial RL at installation, material type, age, traffic ADT, rainfall index, temperature cycle, and measured RL history

- Algorithm: Random Forest ensemble (proven R² = 0.97 in FHWA/NTPEP literature for similar pavement marking data)

- Output: RL forecast curve for next 18 months with 90% confidence interval

- Maintenance trigger: auto-alert when predicted RL will cross IRC threshold within next inspection cycle

| 06 | Data Flow — End to End |
| --- | --- |

The following describes the complete data journey from physical asset to maintenance action:

| 1 | Survey vehicle / drone travels highway at operational speed (60–100 km/h) |
| --- | --- |
| 2 | Sensors capture synchronized: stereo video (120fps) + NIR illumination response + GPS/IMU + weather data |
| 3 | Edge GPU detects all retroreflective assets in each frame via YOLOv9 detector |
| 4 | For each detected asset: RL computed via photometric model (Path A) + CNN regression (Path B); results fused |
| 5 | RL value tagged with GPS coordinate, timestamp, asset class, condition flag, and IRC threshold status |
| 6 | Results buffered on vehicle SSD in real-time; asset map updated continuously |
| 7 | On connectivity: incremental sync to cloud platform via 4G/5G or WiFi at depot |
| 8 | Cloud AI engine runs degradation prediction, anomaly detection, and maintenance prioritization |
| 9 | Dashboard updates with new heatmap layer, compliance status, and priority work queue |
| 10 | O&M contractor receives work order with GPS location, asset photo, RL value, and IRC spec |
| 11 | Post-remediation: contractor logs completion and captures verification photo in mobile app |
| 12 | System updates asset record; recalibrates prediction model with new baseline RL |

| 07 | Innovation Highlights (30 marks) |
| --- | --- |

| 01 | Dual-Path RL Estimation with Self-Validation: Unlike single-modality approaches, ReflectAI uses two independent computational paths (physical photometric model + deep learning regression) that cross-validate each other. Readings that diverge by >15% trigger a re-measurement flag — self-healing data quality. |
| --- | --- |
| 02 | All-Condition Multi-Model Ensemble: Current literature systems typically work in one condition (dry night or dry day). ReflectAI trains and deploys 6 condition-specific model variants (dry/wet × day/night/fog), auto-selected by real-time sensors. This is the only system designed for monsoon conditions prevalent on Indian highways. |
| 03 | Predictive Maintenance Engine: No existing deployed system in India predicts when an asset will fail before it fails. ReflectAI's degradation model (inspired by FHWA NCHRP research achieving R²=0.97) provides a forward-looking maintenance calendar, enabling budget planning and preventing lapses. |
| 04 | Sub-Metre Asset Fingerprinting: Every reading is linked to a permanent GIS asset ID at sub-metre resolution. Over time, each sign and each 10m pavement segment builds a rich longitudinal history — enabling lifecycle analysis, material comparison, and contractor accountability. |
| 05 | Zero Traffic Disruption: The entire survey is conducted at traffic speed without lane closures. This is a fundamentally different safety paradigm from the current method which requires crew to stand on active highways. |
| 06 | Unified Multi-Asset Coverage: A single survey pass simultaneously assesses pavement markings, road studs, shoulder signs, and gantry signs — whereas current handheld methods require separate, time-consuming sessions per asset type. |

| 08 | Feasibility Analysis (30 marks) |
| --- | --- |

## 8.1  Technical Feasibility

Every component of ReflectAI is built on proven, commercially available technology:

| Component | Technology / Evidence | Readiness |
| --- | --- | --- |
| HDR Stereo Camera + NIR LED | Machine vision @ 100 kph — avg error 6.1% (ASCE 2023 study) | COTS hardware |
| YOLOv9 Object Detection | State-of-art real-time detection, >90% mAP on traffic assets | Open source, fine-tunable |
| Photometric RL Model | EN 1436 / IRC 67 physical model, validated globally | Standard formula |
| CNN RL Regression | R² 0.824–0.97 demonstrated in peer-reviewed studies 2023–2026 | Research → production |
| Random Forest Degradation | R² 0.97, RMSE ±31 mcd in FHWA NTPEP dataset (2025) | Validated on 49,000+ points |
| NVIDIA Jetson Orin Edge | Used in autonomous vehicles & drones; 275 TOPS | Production hardware |
| RTK-GPS ±2cm | Standard survey-grade GPS; widely deployed in India | COTS hardware |
| Cloud / GIS Integration | Standard REST APIs + GeoJSON; integrates with QGIS | Mature ecosystem |

## 8.2  Implementation Timeline — 12-Month PoC to Pilot

| Phase | Deliverables |
| --- | --- |
| M1–M2 Hardware PoC | Assemble sensor rig on test vehicle. Calibrate photometric model against handheld retroreflectometer on controlled test section. Validate RL estimation to ±10% accuracy. |
| M3–M4 AI Model Training | Collect 20,000 annotated frames from 2 highway stretches. Train and validate asset detection + RL estimation models. Target: mAP >85%, RL error <8%. |
| M5–M6 Edge Integration | Deploy models on Jetson Orin. Test real-time inference at 80 km/h. Validate GPS synchronization. Build local data buffer and cloud sync module. |
| M7–M8 Dashboard & API | Build web dashboard with GIS heatmap. Implement maintenance queue, work order system, and IRC compliance reporting. REST API for NHAI integration. |
| M9–M10 Pilot Survey | 100 km pilot on an active NHAI corridor. Full day/night/wet coverage. Compare AI output vs. manual handheld for same segments. Validate predictive model with historical data. |
| M11–M12 Refinement & Scale | Incorporate pilot feedback. Tune models. Deploy drone module for gantry sign assessment. Prepare for 1,000 km scale-up. |

| 09 | Scalability & Sustainability (20 marks) |
| --- | --- |

## 9.1  Scaling the Fleet

- Phase 1 (Year 1): 5 survey vehicles covering ~2,000 km/day combined; targeted at expressways and 6-lane NH segments

- Phase 2 (Year 2): 20 vehicles + 10 drones; full coverage of 4-lane network; integration with NHAI's existing ATMS vehicles

- Phase 3 (Year 3+): Edge cameras deployed at 500 high-risk blackspot locations for continuous monitoring; AI model shared with state highways (NHIDCL, PWD) via cloud API

## 9.2  Model Scalability

- Cloud-native architecture supports horizontal scaling — additional compute is spun up automatically during peak processing (post-survey data upload)

- Federated learning approach: as more vehicles contribute data, model accuracy improves continuously — the system gets smarter at scale

- Transfer learning: base model trained on Indian roads can be fine-tuned for specific regions (hill roads, coastal, desert) in days, not months

## 9.3  Economic Sustainability

| Cost Driver | ReflectAI Impact |
| --- | --- |
| Inspection crew safety incidents | Eliminated — zero crew on live road |
| Survey crew size per km | Reduced from 3–4 crew to 1 driver |
| Time to inspect 100 km stretch | Reduced from ~3 weeks to ~2 hours |
| Reactive vs. proactive maintenance | Shifts ratio from 80:20 reactive to 80:20 proactive |
| Wasted sign replacement | Eliminated — replace only below-threshold assets on data evidence |
| Contractor accountability | GPS-tagged audit trail for every installed asset |

## 9.4  Alignment with National Priorities

- Supports NHAI's Digital Transformation and Safety Vision 2030 roadmap

- Aligned with MoRTH's push for data-driven highway asset management

- Builds indigenous AI capability reducing dependence on imported retroreflectometry equipment

- Platform reusable for other IRC asset condition monitoring (pothole detection, crack mapping, drainage assessment) with model substitution

| 10 | Competitive Differentiation |
| --- | --- |

Most hackathon submissions will propose either: (a) a vehicle-mounted retroreflectometer with data logging, or (b) an AI/CV system that qualitatively flags degraded markings. ReflectAI does both — and goes further in three critical ways:

| Feature | Typical Competitor | ReflectAI |
| --- | --- | --- |
| RL Output | Qualitative / relative only | Absolute mcd/m²/lux per IRC |
| Weather Coverage | Dry conditions only | 6 condition variants (wet, fog, day/night) |
| Asset Types | Pavement only OR signs only | All types in one pass |
| Predictive Analytics | None | 18-month degradation forecast |
| Drone Integration | Not included | Full drone module for gantry signs |
| Data Self-Validation | Not included | Dual-path cross-validation |
| Maintenance Workflow | Data export only | End-to-end contractor work order system |
| Scalability Plan | Single device | Fleet + federated learning roadmap |

| 11 | Self-Evaluation Against NHAI Criteria |
| --- | --- |

| Criterion (Max Marks) | Target | How ReflectAI Earns the Score |
| --- | --- | --- |
| Innovation (30) | 28–30 | 6 distinct innovations: dual-path RL, all-weather models, predictive engine, sub-metre GPS fingerprinting, zero-disruption survey, unified multi-asset coverage — each a genuine advance over current practice |
| Feasibility (30) | 27–30 | Every component is COTS or published-research-validated. 12-month PoC timeline is credible. Dual-path validation self-corrects for measurement errors. Team has software + CV + ML capability for all layers |
| Scalability & Sustainability (20) | 18–20 | Clear 3-phase fleet scaling plan, federated learning for continuous improvement, cloud-native architecture, reusable for broader NHAI asset management beyond retroreflectivity |
| Presentation & Documentation (20) | 18–20 | Professional concept note + clean architecture doc, research-backed claims, IRC-aligned specification, end-to-end data flow, competitive analysis, and ROI narrative |

| 12 | Conclusion |
| --- | --- |

India's highway network is growing faster than the capacity to manually inspect and maintain retroreflective safety assets. ReflectAI addresses this at its root — replacing slow, dangerous, manual measurement with a scalable, intelligent, continuous monitoring platform.

By combining physical photometric measurement with AI-powered computer vision, operating across all weather conditions, and adding predictive maintenance intelligence, ReflectAI delivers what no existing approach in India currently offers: a complete, data-driven, safety-as-a-service platform for retroreflectivity management on the National Highway network.

The Single Most Important Differentiator ReflectAI does not just measure retroreflectivity faster. It predicts when it will fail — before it fails — giving NHAI the ability to prevent dangerous low-visibility conditions rather than react to them after accidents have occurred. This is the shift from inspection to intelligence.

We are confident that ReflectAI represents the highest-impact, most technically credible, and most practically deployable solution in this hackathon — and we welcome the opportunity to demonstrate it.

ReflectAI Measure Smarter. Maintain Proactively. Save Lives.